SCRUB — Current Technical Specifications

Surface Cleaning Robot for Urban Bodies of Water  —  Prototype v0.6

AASK Technologies · RV College of Engineering, Bengaluru · April 2026

Document Ref: AASK/SCRUB/SPEC/2026/v0.6

Technology Readiness Level:  TRL-4  (Validated in lab / pool environment — 20+ hours accumulated runtime)

1.  Physical Dimensions & Mass

| Parameter | Value / Notes |
| --- | --- |
| Length | 1.0 m (optimised for urban lake stretches) |
| Width | 0.65 m |
| Height | 0.40 m |
| Dry Weight | 25 kg (lightweight for two-person transport and deployment) |
| Total Operational Mass | 75 kg (25 kg dry + 50 kg payload / debris capacity) |
| Hull Draft at Full Load | ~11.5 cm (calculated: 0.075 m³ displaced / 0.65 m²) |
| Hull Material (current) | PVC pipe frame + tyre float (v0.6 prototype) |
| Hull Material (planned) | GRP / Carbon-Kevlar hybrid composite (production version) |
| Buoyancy Capacity | 75 kg operational load confirmed in pool testing |

2.  Propulsion System

| Parameter | Specification |
| --- | --- |
| Motor Type | Brushless DC (BLDC) ×2 |
| Power per Motor | 12 W (upgrade to 20 W recommended — see Engineering Analysis) |
| Steering Mechanism | Differential thrust (independent port and starboard drives) |
| Maximum Speed | 0.7 m/s |
| Current Rating | Stable in moderate lake currents |
| Drive Architecture | Independent port & starboard ESC control via STM32 PWM |
| ESC Recommended | VESC 4.12 or FSESC4.20 (48V, 50A rated) |
| Propeller Status | Replacement with screw-mount propellers planned pre-demonstration |
| Propeller Guard | 3D-printed debris shroud in design — prevents weed jamming on shaft |

3.  Power System

| Parameter | Specification |
| --- | --- |
| Battery Chemistry | LiFePO₄ (safe, 2000+ cycle life) |
| Battery Configuration | 16S1P (16 cells in series) |
| Nominal Pack Voltage | 51.2 V (spec states 48V bus) |
| Battery Capacity | 20 Ah |
| Pack Energy | 1,024 Wh total / 819 Wh usable (80% DoD) |
| Estimated Pack Mass | ~7.9 kg (at 130 Wh/kg LiFePO₄) |
| Solar Panel | 25 W monocrystalline (Vmp ≈ 19.5 V, Imp ≈ 1.28 A) |
| Solar Architecture | Boost MPPT required (19.5 V panel → 51.2 V bus) |
| Autonomous Runtime | 3–4 hours continuous (battery primary; solar extends runtime) |
| Shore Charging | 5A charger, ~4 hours 0→100% |
| Continuous System Load | 63.9 W (all subsystems active at cruise) |
| Continuous Bus Current | 1.33 A @ 48 V (very low C-rate — 0.067C) |
| BMS | Daly 16S 30A LiFePO₄ BMS (OVP / UVP / OCP / cell balancing) |
| MPPT Controller | Victron SmartSolar MPPT 75/15 (or CN3791 custom) |

4.  Waste Collection Mechanism — Conveyor Assembly

| Parameter | Specification |
| --- | --- |
| Mechanism Type | Inclined conveyor belt — front-mounted at bow |
| Belt Material | Stainless Steel SS316L mesh (corrosion-resistant, food-grade) |
| Conveyor Angle | 14° (incline from water surface to collection bin) |
| Mesh Aperture | 2 mm (captures waste; prevents entry of fish and aquatic life) |
| Operating Depth | 0–50 mm (shallow surface zone; avoids fish nursery zones and bed) |
| Collection Capacity per Cycle | 20 kg (design target) |
| Processing Throughput | 10–15 kg/min |
| Drive Roller Torque Required | 4.56 N·m (calculated from first principles) |
| Conveyor Motor | 12V DC gearmotor, 30 RPM output, ≥25 W, IP65 rated |
| Drive Roller Grip | EVA / NBR foam wrap — PENDING fitting |
| Mesh Tensioner | 3D-print/metal hybrid assembly — IN PROGRESS |
| Lateral Anti-Slip Disks | Press-fitted flange guides on front lateral member — PENDING TEST |
| Side Containment Panels | 2 mm fibreglass triangular panels — PENDING fabrication |
| Mesh Liner Sheet | Solid backing sheet between mesh runs (for algae/moss) — PENDING |

5.  Debris Detection — AI Vision System

| Parameter | Specification |
| --- | --- |
| ML Model | YOLOv8 (real-time object detection) |
| Detection Accuracy | 90% mAP (validated in controlled water environment) |
| Inference Platform | Raspberry Pi 5 (8 GB) — on-device, no cloud dependency |
| Camera | 5 MP front-facing USB camera module |
| Inference Latency | <80 ms per frame (sufficient for 0.4 m/s cruise) |
| Detected Classes | Floating plastics · Thermocol · Organic matter · Clear water |
| Training Dataset | SCRUB-specific dataset — augmented for lake environment |
| Vision Role | Guides navigation to high-density debris patches; informs path replanning |

6.  Water Quality Sensor Suite

| Sensor | Parameter Measured / Accuracy |
| --- | --- |
| pH Probe | pH level — ±0.2 pH units |
| Turbidity Sensor | Turbidity (NTU) — ±5 NTU |
| Temperature Probe | Water temperature — ±0.5°C |
| TDS Sensor | Total Dissolved Solids (mg/L) |
| MQ-135 Gas Sensor | Ambient air quality / surface gas detection |
| DHT11 | Air temperature and relative humidity |
| GPS (NEO-M8N) | Position tagging for all sensor logs — GPS + GLONASS |
| Ultrasonic ×2 (HC-SR04) | Obstacle detection — triggers motor halt within safety distance |
| Logging Rate | 1 Hz (GPS-tagged; each reading geo-stamped) |
| Validation | Cross-validated against laboratory standard instruments |

Monitoring Advantage:  SCRUB provides continuous, spatially distributed water quality data across the entire lake surface on every cleaning run — replacing weekly manual sampling at fixed points. Pollution events (sewage inflow, chemical spills) can be detected within minutes rather than weeks.

7.  Compute, Navigation & Control Architecture

7.1  Embedded Computing

| Component | Role / Specification |
| --- | --- |
| Raspberry Pi 5 (8 GB) | Primary mission computer — ML inference, GPS navigation, IoT uplink, cloud dashboard |
| RPi5 Power | 5V / 5A via USB-C PD (25W max; active cooler required) |
| RPi5 Active Power | 12–15 W (during ML inference) |
| STM32G431CB | Real-time MCU — motor PWM (20 kHz), sensor ADC, safety watchdog |
| STM32 Supply | 3.3 V @ 150 mA (0.5 W from 5V rail via AMS1117) |
| 5 MP Camera | Vision — debris detection + obstacle monitoring (USB / CSI) |
| Architecture | Dual-CPU: RPi5 (Linux, high-level) + STM32 (bare-metal, hard real-time) |
| Watchdog | STM32 IWDG: 4-second timeout; if RPi5 silent → MCU commands E-stop |

7.2  Navigation & Autonomy

| Feature | Detail |
| --- | --- |
| Path Planning | Grid-based autonomous navigation with GPS waypoints |
| Obstacle Avoidance | Proximity sensors halt movement on obstruction detection (validated in complex layouts) |
| Operation Mode 1 | Fully autonomous AI navigation (GPS + sensor fusion + ML) |
| Operation Mode 2 | Manual remote control (wireless RC link; switchable real-time) |
| Collision Validation | Obstacle avoidance tested in complex obstacle environments |
| Dead Reckoning | IMU + encoder odometry fallback during GPS outage (planned v1.0) |

8.  Connectivity & IoT Platform

| Parameter | Specification |
| --- | --- |
| Primary Connectivity | 4G LTE modem (SIMCom SIM7600G-H) |
| Secondary Connectivity | Wi-Fi (ESP32 or RPi5 onboard) |
| Data Upload | Real-time cloud dashboard (WebSocket + MQTT protocol) |
| Dashboard — Live Video | 5 MP camera feed streamed over 4G |
| Dashboard — GPS Tracking | Real-time robot position on map overlay |
| Dashboard — Water Quality | Live sensor readings with threshold alerts and heatmaps |
| Dashboard — Bin Fill Alert | Automated alert when collection bin approaches capacity |
| Offline Mode | Local SQLite buffer during connectivity loss; uploads on reconnection |

9.  Power Distribution Architecture

| Rail | Load / Converter |
| --- | --- |
| 48V (direct from battery) | BLDC motor ESCs ×2 — no conversion |
| 12V (48V→12V buck) | 4G LTE modem · Conveyor H-bridge · Auxiliary — Meanwell SD-25D-12 |
| 5V (48V→5V buck + PD) | Raspberry Pi 5 via USB-C PD — Pololu D36V50F5 or equiv. |
| 3.3V (5V LDO) | STM32 · GPS · Wi-Fi module — AMS1117-3.3 on PCB |
| Solar MPPT (boost) | 19.5 V panel → 51.2 V battery — Victron 75/15 or CN3791 |
| Total Conversion Loss | ~2.7 W (at 92% converter efficiency) |

10.  Mechanical Upgrades — Current Development Status  (April 2026)

| Upgrade Item | Status | Description |
| --- | --- | --- |
| Mesh Tensioner | IN PROGRESS | 3D-print/metal hybrid saddle clamp + M-rod assembly. Clamps to base chassis cross-member to eliminate mesh slack and reduce vibration noise. |
| Lateral Anti-Slip Disks | PENDING TEST | Press-fitted flange disks on front lateral member to constrain mesh from lateral drift. Test protocol defined. |
| Side Containment Panels | PENDING | 2mm fibreglass triangular panels to replace cardboard — prevents waste spilling off belt sides before reaching bin. |
| Mesh Liner Sheet | PENDING | Solid plastic sheet between mesh runs — solid backing surface retains algae/moss that drains through stainless mesh perforations. |
| Drive Roller Grip Layer | PENDING | EVA/NBR foam wrap on drive roller for friction. Prevents mesh slippage under wet load. Wrap <3–4mm to preserve roller geometry. |
| Propeller Replacement | PRE-DEMO | Screw-mount hub propellers — balanced, correct pitch for BLDC RPM. Replaces current propellers for improved thrust. |
| Propeller Guard / Shroud | PRE-DEMO | 3D-printed PETG/ASA debris shroud around each propeller shaft. Flow-through geometry. Prevents weed wrap — critical failure mode during demos. |
| Stepper Motor (Conveyor) | OPTIONAL | NEMA stepper + A4988 driver from lab rig — precise belt speed control without encoder feedback. Optional upgrade. |
| Waste Collection Drawer | OPTIONAL | Removable tray at conveyor discharge end — one-handed extraction at shore, reduces secondary contamination. |
| Electronics Enclosure | OPTIONAL | IP54 3D-printed ASA enclosure for all electronics under top panel. Splash protection + heat management. |

11.  Key Performance Metrics

| Metric | Value |
| --- | --- |
| Waste collected per cycle | 20 kg (target; 3–4 kg validated in v0.5 prototype) |
| Collection throughput | 10–15 kg/min |
| Annual waste diverted per unit | 600–1,200 kg (at 30–60 operating cycles/year) |
| CO₂ avoided (10-unit fleet/year) | ~30 tonnes |
| Sensor logging rate | 1 Hz (GPS-tagged) |
| Debris detection accuracy | 90% (controlled water; lake generalisation in development) |
| Maximum operational speed | 0.7 m/s |
| Autonomous runtime (battery + solar) | 3–4 hours/day |
| Target unit cost at 50+ unit scale | ₹1,50,000 |
| Current prototype cost | ₹3,36,400 (incl. chassis, electronics, field testing) |
| Pool test runtime accumulated | 20+ hours |

12.  Supply Chain — Indigenous Sourcing  (Bengaluru & India)

| Supplier / Platform | Components Sourced |
| --- | --- |
| Robu.in (online) | BLDC motors, ESCs, motor controllers, embedded modules, sensors |
| Sunroom Solar (India) | Solar panels, power management modules |
| Lion Circuits (Bengaluru) | Custom PCB fabrication, SMT assembly, testing |
| Local Bengaluru fabricators | Stainless steel SS316L conveyor frame, structural pipe fittings |
| 3D Printzkart (Bengaluru) | GRP / Carbon-Kevlar composite hull — production chassis (contracted) |
| Robu.in / Amazon India | LiFePO₄ battery cells, BMS, DC-DC converters |
| CATL / EVE / Lishen (via distributors) | LiFePO₄ 20Ah prismatic cells for 16S battery pack |

100% Indigenous Manufacturing:  SCRUB is designed for fully Indian manufacturing using established domestic suppliers. All components are procurable within India, making SCRUB manufacturable, scalable, and serviceable in India at a fraction of the cost of imported alternatives.

14.  Testing & Validation Results

14.1  Pool Testing Summary

| Test Parameter | Result / Notes |
| --- | --- |
| Total Runtime Accumulated | 20+ hours across multiple sessions |
| Maximum Continuous Runtime | 4.5 hours (battery + solar extension) |
| Waste Collection Efficiency | 85% capture rate for floating plastics in controlled tests |
| Obstacle Avoidance | 100% success in complex obstacle courses |
| Water Quality Accuracy | ±5% deviation from lab instruments |
| GPS Navigation Accuracy | ±2m positional accuracy in open water |
| ML Detection Accuracy | 90% mAP in pool environment |
| Propulsion Reliability | No motor failures; propeller weed-jamming mitigated |
| Hull Integrity | No leaks or structural failures under load |
| Power System Stability | Stable voltage rails; no brownouts during operation |

14.2  Field Testing Plans

Upcoming Demonstrations:  SCRUB v0.6 will undergo field testing in urban lake environments (Bengaluru) starting May 2026. Key objectives include real-world debris collection efficiency, sensor data validation against municipal water quality standards, and autonomous navigation in uncontrolled environments.

- Lake surface mapping and GPS waypoint validation

- Debris collection in natural water conditions (weeds, currents, wind)

- IoT dashboard real-time monitoring and alert system testing

- Solar charging performance in variable weather conditions

- Mechanical upgrades validation (mesh tensioner, propeller guards)

14.3  Risk Mitigation

| Risk | Mitigation Strategy |
| --- | --- |
| Propeller Weed Jamming | Propeller guards + shroud design; tested in weed-heavy environments |
| Mesh Slippage | Drive roller grip layer + tensioner assembly |
| Electronics Water Ingress | IP54 enclosure + conformal coating on PCBs |
| GPS Signal Loss | Dead reckoning fallback with IMU + odometry |
| Battery Over-Discharge | BMS protection + low-voltage cutoff |
| ML Model Generalisation | Additional training data from field conditions |

15.  Future Roadmap & Scaling

15.1  Version Milestones

| Version | Timeline / Key Features |
| --- | --- |
| v0.7 (Current) | April–May 2026: Mechanical upgrades completion, field demo prep |
| v1.0 (Production Ready) | June–August 2026: Composite hull, full IP54 electronics, dead reckoning |
| v1.5 (Fleet Optimisation) | Sep–Dec 2026: Multi-unit coordination, advanced ML models |
| v2.0 (Advanced Features) | 2027: Underwater capabilities, chemical spill detection |

15.2  Scaling Strategy

Market Entry:  Initial deployment in Bengaluru municipal lakes, followed by expansion to other Indian cities. Target 50+ units by 2027, creating 200+ direct jobs in manufacturing and operations.

- Partnership with municipal corporations for pilot programs

- Franchise model for regional manufacturing and maintenance

- Export potential to Southeast Asia and Middle East markets

- R&D collaboration with IITs and DRDO for advanced features

16.  Intellectual Property Status

| Filing | Details |
| --- | --- |
| Patent 1 — Filed | System patent — vision pipeline, autonomous navigation, IoT integration ¦ Filed Sep 2024 ¦ PUBLISHED |
| Patent 2 — In Development | Design patent — weed-puller servo-deployable mechanism ¦ Target filing: June 2026 |
| Patent 3 — In Development | Utility patent — custom PCB architecture for aquatic robotics ¦ Target filing: July 2026 |
| DPIIT Startup India | Registration planned April 2026 — unlocks tax benefits and fast-track IP processing |

SCRUB Technical Specification  v0.6  ·  AASK Technologies  ·  RVCE Bengaluru  ·  April 2026

Contact: kpurwar1234@gmail.com  ·  +91 7007633541  ·  aask-kappa.vercel.app